# VLAN and VLAN Configuration

## 1. Introduction to VLAN

A Virtual Local Area Network (VLAN) is a logical grouping of devices within a physical network. VLANs allow network administrators to segment a LAN without changing the physical topology.

### Why VLANs are Needed

- Reduce broadcast traffic

- Improve network security

- Better network management

- Logical grouping of users (e.g., Accounts, HR, IT)

## 2. Types of VLAN

### a) Default VLAN

- VLAN ID: 1

- All switch ports belong to VLAN 1 by default

- Not recommended for user traffic (security risk)

### b) Data VLAN

- Used to carry user-generated data

- Example: VLAN 10 for Accounts

### c) Management VLAN

- Used for switch management (SSH, Telnet, SNMP)

- Example: VLAN 99

### d) Native VLAN

- Used on trunk links

- Carries untagged traffic

- Default is VLAN 1

### e) Voice VLAN

- Used for IP phones

- Ensures QoS (Quality of Service)

## 3. VLAN ID Range

- Normal VLANs: 1 – 1005

- Extended VLANs: 1006 – 4094

## 4. VLAN Membership Methods

### a) Static VLAN

- Manually assigned

- Most commonly used

### b) Dynamic VLAN

- Assigned using VMPS (rarely used)

## 5. VLAN Tagging (IEEE 802.1Q)

- Adds a 4-byte tag to Ethernet frames

- Identifies VLAN ID

- Used on trunk links

## 6. Access Port vs Trunk Port

### Access Port

- Carries traffic for one VLAN only

- Used for end devices (PCs, printers)

### Trunk Port

- Carries traffic for multiple VLANs

- Used between switches or switch–router

## 7. VLAN Configuration (Cisco Switch)

### Step 1: Create VLAN

Switch> enable

Switch# config t

Switch(config)# vlan 10
Switch(config-vlan)# name Accounts
Switch(config-vlan)# exit

### Step 2: Assign VLAN to Single Access Port

Switch# config t

Switch(config)# interface fastEthernet0/1 or (int fa0/1)
Switch(config-if)# switchport access vlan 10
Switch(config-if)# exit

### Step 2.1: Assign VLAN to Multi Access Port

Switch# config t

Switch(config)# interface range fastEthernet0/1-5 or (int range fa0/1-5)
Switch(config-if)# switchport access vlan 10
Switch(config-if)# exit

### Step 3: Move Interface form VLAN 10 to Native VLAN

Switch# config t

Switch(config)# interface range fastEthernet0/1-5 or (int range fa0/1-5)
Switch(config-if)# switchport access vlan 1
Switch(config-if)# exit

### Step 3: Configure Trunk Port

Switch# config t

Switch(config)# interface fastEthernet0/1 or (int fa0/1)
Switch(config-if)# switchport mode trunk

Switch(config-if)# end

### Step 4: Router Configure for Trunk Port

Switch# config t

Switch(config)# interface gigabitEthernet0/0 or (int gig0/0)
Switch(config-if)# no shoutdown

Switch(config-if)# end

### Step 5: IP Assign in Router Interface

Switch# config t

Switch(config)# interface gigabitEthernet0/0.1 or (int gig0/0.1)
Switch(config-if)# encapsulation dot1q 10

Switch(config)# interface gigabitEthernet0/0.1 or (int gig0/0.1)
Switch(config-if)# encapsulation dot1q 10

Switch(config-if)# ip add 192.168.10.1 255.255.255.0

Switch(config-if)# end

## 8. Verify VLAN Configuration

### Show VLAN Information

Switch# show vlan brief

### Show Trunk Information

Switch# show interfaces trunk

## 9. Inter-VLAN Communication

- VLANs cannot communicate directly

- Requires:

  - Router (Router-on-a-Stick)

  - Layer 3 Switch

### Router-on-a-Stick Concept

- Single router interface

- Multiple sub-interfaces

- Each sub-interface handles one VLAN

## 10. Advantages of VLAN

- Reduced broadcast domain

- Improved performance

- Enhanced security

- Scalability

## 11. Disadvantages of VLAN

- Initial configuration complexity

- Requires VLAN-aware devices

## 12. Common VLAN Troubleshooting Commands

show vlan brief
show interfaces switchport
show running-config
show ip interface brief